| *AUTOMATIC SINTERING MACHINE MOD. 18STV, WITH CONTROLLED ATMOSPHERE, ONE STATION, 120Kva THREE-PHASE, 95 TON. |  |
| --- | --- |
| Main features |  |
|  Sintering machine suitable for the production of average and large size segment series, continuous rim blades, grinding wheels for glass and other diamond tools. |  |
|  Mould heating system for direct passing of direct current (Joule effect). |  |
|  Sintering with steel and graphite moulds. |  |
|  Sintering under controlled atmosphere by means of vacuum/inert gas programmable wash. |  |
|  Type K thermo-couple for operating temperatures up to 1000°C. |  |
|  Plates cooling, transformer and vacuum chamber with flow visualisation and water temperature. |  |
|  120kVA three-phase transformer, (max. power supply is limited electronically to 210A) by switch for 3 different tensions between the electrodes(maximum power limit for heating). |  |
|  Controller program with cycles divisible into 12 steps. each step includes: |  |
|  temperature |  |
|  pressure |  |
|  max. power limit |  |
|  heating time |  |
|  holding time |  |
|  Compression force controlled electronically (loop system) |  |
|  Process data visualisation (set and real) and alarms. |  |
| Options |  |
|  Bar code for selecting sintering programs. |  |
|  Temperature control 350-1300°C by infra-red rays pyrometer |  |
|  Type R thermo-couple for operating temperatures up to 1200°C. |  |
|  Additional control thermo-couples. |  |
|  Segment height control while sintering takes place (linear transducer on cylinder, resolution of 0,01mm) |  |
|  Minimum compression force reduction system (from 4200 to 2400kg). |  |
|  Data acquisition system on external PC. |  |
|  Electrodes in graphite and amorphous carbon. |  |
|  Graphite plates. |  |
|  Graphite moulds. |  |
|  Aluminium frames. |  |
|  Prearrangement for modem installation for remote assistance. |  |
| Technical data |  |
|  Max. sintering area | 180 cm2 (850°C 350kg/cm2) |
|  Power supply | 380 / 400V 3phs 50Hz |
|  Max. power consumption | 120 kVA (210 A) |
|  Transformer | 120 kVA trifase |
|  Compression force (adjustable) | 4200 - 95000 kg |
|  Electrode dimensions | 225 x 225x60(h)mm |
|  Plate dimensions | 400 x 400x60(h)mm |
|  Aluminium frames dimensions | 238 x 188mm |
|  Max. mould height | 175mm |
|  Gas consumption | 15 – 25 l/min N2/Ar2 (max. 5% H2) |
|  Air consumption (filtered) | 2 l/min 6 bar |
|  Industrial water consumption (approx.) | 80 - 100 l/min 2–6 bar 10-20°C |
|  Colour | Choice |
|  Overall size (approx.) | 2050x2600x2100(h) mm |
|  Weight (approx.) | 3500 kg |

* MGM  reserve itself the right to change the characteristics described in this document without previous notice.